Article information

Article title

Enhanced Bearing Health Indicator Extraction Using Slope Adaptive Signal Decomposition for Predictive Maintenance.

Authors

Dev Bhanushalia,Pooja Kamata, Harsh Dhimanb

Affiliations

a Symbiosis Institute of Technology, Symbiosis International (Deemed University), Pune, Maharashtra, India

bErnst & Young LLP India

Corresponding author’s email address and Twitter handle

Pooja.kamat@sitpune.edu.in

Keywords

Predictive Maintenance; Bearing Health Monitoring; Signal Processing; Slope Adaptive Signal Decomposition (SASD); Remaining Useful Life (RUL); Machine Health Indicator

Related research article

For a published article:

Abstract

This study introduces the Slope Adaptive Signal Decomposition (SASD) algorithm, a novel method for extracting enhanced intrinsic machine health indicators from vibration data. Leveraging advanced signal processing techniques such as dynamic Savitzky-Golay filtering, segmentation, and trend-based recalibration, SASD achieves superior noise attenuation while preserving critical trends. Applied to the PRONOSTIA platform’s bearing datasets, SASD produces refined health indicators suitable for predictive maintenance tasks, including Remaining Useful Life (RUL) estimation. The extracted features are evaluated using deep learning models like GRU, LSTM, and hybrid architectures, as well as conventional regression approaches, demonstrating SASD's effectiveness in improving prediction accuracy. Among these, GRU exhibits best performance with R2 score above 0.96 across all 3 operating conditions for various bearings. This method bridges the gap between signal processing and data-driven prognostics, enabling robust bearing health monitoring under varying operational conditions. In the future, the SASD framework will be extended to other industrial datasets to benchmark its generalizability across different operating conditions. Additionally, integrating real-time data streaming capabilities and edge computing deployments can further improve the scalability for real-world predictive maintenance applications.

Highlights:

- Dynamic signal smoothing and segmentation for enhanced trend extraction.

- SASD-based health indicators outperform traditional preprocessing techniques.

- Validated through RUL prediction using both statistical and deep learning models.

Graphical abstract

Specifications table

| Subject area | Engineering |
| --- | --- |
| More specific subject area | Deep Learning |
| Name of your method | Slope Adaptive Signal Decomposition for bearing life estimation |
| Name and reference of original method | P. Nectoux et al., “PRONOSTIA : An experimental platform for bearings accelerated degradation tests,” IEEE International Conference on Prognostics and Health Management, PHM’12., Jun 2012, Denver, Colorado, United States., pp. 1–8, 2012 [1] |
| Resource availability | Data: Data available on request Software: Google Colab, Scipy library |

Background

Condition-based monitoring (CBM) has undergone profound changes from simple, traditional manual inspections to advanced automated methodologies [2], [3]. Initial CBM techniques were based on simple visual inspections and threshold-based parameter evaluations, like vibration and temperature, for determining the health of equipment [4], [5]. Deep learning has recently emerged as a game-changer in machinery health monitoring, especially when associated with data-driven approaches, such as Remaining Useful Life (RUL) prediction [6]. Deep learning models can independently learn to extract features and map complex temporal dependencies, making them highly effective in applications such as anomaly detection and RUL estimation [7], [8]. Recently, such models demonstrated superiority over classic techniques using predictive maintenance by learning complicated patterns in large datasets of operationally observed signals [9]. The accuracy and reliability of these models depend only on the quality of the input signals[10]. The two essential practices governing successful predictive maintenance are health monitoring and estimation of the Remaining Useful Life (RUL) for machine components [11], [12]. Bearings as a part of rotating machinery determine the reliability and efficiency of industrial systems [13]. Meanwhile, vibration signal analysis emerges as a powerful tool for diagnosing bearing health, but issues such as noise interference and trend distortion deter the extraction of meaningful indicators [14]. To achieve this, this paper introduces the Slope Adaptive Signal Decomposition (SASD) algorithm, a novel signal processing method that is capable of extracting enhanced health indicators from raw vibration data. The SASD algorithm is rich in dynamic smoothing, segmentation, and trend-based recalculation, ensuring noise attenuation while preserving trends. This method, applied to the PRONOSTIA platform[1], generates reliable health indicators. Advanced machine learning models then evaluate these indicators for RUL estimation. This research aims to fill the gap between signal preprocessing and data-driven maintenance strategies by offering a scalable solution to bearing health monitoring with effectiveness in areas. Techniques like proposed SASD can bridge this gap, as they bring enhanced health indicators with advanced signal preprocessing, thereby paving the path for improved outcomes of CBM in different operational conditions. The SASD algorithm framework is showcased in Figure 1.

Figure 1: Slope Adaptive Signal Decomposition (SASD) algorithm

Method details

This section introduces the experimental description, dataset preparation as well as the proposed novel Slope Adaptive Signal Decomposition algorithm for extracting an enhanced intrinsic machine health indicator from vibration data by implementing a series of advanced signal processing techniques[15]. The core algorithm involves a succession of preprocessing, segmentation, trend extraction, and signal modification operations. Each stage is mathematically formulated to balance noise attenuation and trend preservation. The pseudocode corresponding to this method is outlined in Algorithm 1 and is elaborated upon in detail in the subsequent subsections.

Experimental description and feature extraction

The bearing vibration signals used in this manuscript were collected from the PRONOSTIA platform [1]. There are a total of 17 bearing datasets across 3 different operating conditions as described in Table 1. The vibration data was collected through two accelerometers placed at 90◦ to each other on the bearing’s external race at a sampling frequency of 25.6 kHz.

We start by extracting essential time domain features from the raw vibration signals of bearings. The vibration data is subjected to a non-overlapped sliding window mechanism where each window captures a segment of the horizontal and vertical components of the signals captured by the corresponding accelerometers. From each window, a series of statistical features are computed as described mathematically in Table 2. We will then process these extracted features further using our novel SASD algorithm which is described in detail in the next subsection.

TABLE 1: Datasets of PRONOSTIA platform

|  | Condition 1 | Condition 2 | Condition 3 |
| --- | --- | --- | --- |
| Radial load (N) | 4000 | 4200 | 5000 |
| Speed (r/min) | 1800 | 1650 | 1500 |
| Training datasets | Bearing 1–1 | Bearing 2–3 | Bearing 3–1 |
|  | Bearing 1–2 | Bearing 2–5 | Bearing 3–2 |
| Testing datasets | Bearing 1–3 | Bearing 2–1 | Bearing 3–3 |
|  | Bearing 1–4 | Bearing 2–2 |  |
|  | Bearing 1–5 | Bearing 2–4 |  |
|  | Bearing 1–6 | Bearing 2–6 |  |

TABLE 2: Extracted time domain features [20]

| Feature Name | Formula |
| --- | --- |
| Mean (σ) | μ=1Ni=1Nxi |
| RMS | RMS=1Ni=1Nxi2 |
| Variance (σ2) | σ2=1Ni=1Nxi-μ2 |
| Standard Deviation (σ) | σ=1Ni=1Nxi-μ2​ |
| Minimum Absolute Value | minx=minx1,x2,…,xN |
| Maximum Absolute Value | maxx=maxx1,x2,…,xN |
| Peak-to-Peak Amplitude (P2P) | P2P=maxx-minx |
| Impulse Factor | IF=maxxmeanx |

Algorithm 1: Signal Adaptive Slope Decomposition

| Input: | X={X1,X2,…,XN}: Vibration signals over time. λwin: Window factor for segmentation. λpre: Pre-smoothing factor. βthresh: Threshold for slope detection. κelev: Elevation factor for recalibration. λpost: Post-smoothing factor. |
| --- | --- |
| Output: | Sproc={s1,s2,…,sN}: Processed signal. |
| Apply Savitzky-Golay filter for smoothing: | Ssmootht=SGFilterX,Nλpre,2 |
| Determine window length T: | T=max10,Nλwin |
| Segment the smoothed signal into non-overlapping windows: | Wit=Ssmoothti:ti+T |
| For each window Wi, perform linear regression to estimate local trends: | Wregt=βit+αi |
| Recalibrate the signal: | Wrecalt=Wit, if &βi≥0Wit Wregtκelev, &if βi<0 |
| Post-smooth the recalibrated signal: | Wfilteredt=SGFilter Wrecalt, Tλpost, if &βi<βthreshWrecalt, &otherwise |
| Adjust for continuity: | Srecont=Wfilteredt+Sreconti-1+T-1-Wfilteredt0 |
| Compile the processed signal: | HI={Srecon1,Srecon2,…,SreconN} |
| Return: | HI |

Slope Adaptive Signal Decomposition algorithm

Formalization and Multi-pass Savitzky-Golay filtering[16]

Let the raw vibration signal be represented by (st) where (st∈R),  and (t) denotes the time index of the signal over (N) discrete points. To remove the high frequency noise, the signal is preprocessed using a Savitzky-Golay filter iteratively to smoothen the data without distorting the original signal structure. This filter fits a low degree polynomial over a sliding window to approximate underlying signal dynamics. It is essentially a local polynomial smoother that can be mathematically formalized in the following manner. For some window length (wSG), polynomial degree (p) and an interval (It) described as follows.

It=t-wSG2,t+wSG2	(1)

The filter approximates signal (st) at each timestep (t) by fitting a polynomial (Pt∈Pp) where (Pp) is the space of polynomials of degree (p) and minimizing the squared error over the window (It), formally written below.

Pt=argminP∈Pτ∈Itsτ-Pτ2	(2)

Given this, the value of (Pt) at each time (t) yields the smoothed signal. The window size, (wSG) in the SASD algorithm is dynamically adapted based on the size of the signal and a pre-smoothing factor (λpre) in the following manner.

wSG=λpreN	(3)

The need for a dynamically updated window size will be discussed shortly. By applying the SG filter iteratively, we refine the denoising operation progressively through multiple passes. The filtered signal after (n)-th iteration is denoted as (Snt). The iterative process can be mathematically described as follows.

Sn+1t=SGFilterSnt,wSG,p	(4)

where (n=0,1,…,Niter-1) and (Niter) is the number of iterations of the filter. Typically, (p = 2) and (Niter) is set to 5.

Signal segmentation and local trend estimation

Following the denoising step, the filtered signal (Snt) is partitioned into a series of non-overlapping window segments (Wit) for local signal transformation. Just as we previously defined the window size for the Savitzky-Golay filter, the window size for each non-overlapping segment of this step is determined in a similar fashion, which is as follows.

Ti=max10,Nλwin	(5)

This will ensure that the windows are large enough to capture meaningful trends while avoiding over-segmentation that may distort the global structure of the signal. Each segmentation (Wit) is defined as follows.

Wit=Snti:ti+Ti	(6)

where (ti) is the starting index of the (ith) window and (Ti) is the window length. Following this, each window (Wit) is subjected to local trend estimation. We assume that each signal segment roughly follows a locally linear model. This trend is then estimated using ordinary least squares regression, fitting the data within each segment to a linear function as described below.

Wreg,it=βit+αi	(7)

where (βi) and (αi) are the slope and intercepts of the (ith) window segment, and are computed by minimizing the sum of squared residuals in the following manner.

minβi,αit=titi+TiWit-βit+αi2	(8)

We then identify the local trend direction by evaluating the estimated slope (βi). If (βi≥0), the local trend is upward, while (βi<0) indicates a downward trend.

Trend-Based conditional signal modification, Post-recalibration denoising and global signal reconstruction

The estimated regression parameters (βi) and (αi) drive the subsequent modification of the signal segment, specifically for cases where the slope parameter (βi) is negative. The segment undergoes recalibration to emphasize these features in an upward direction. The recalibrated signal, (Wit) can be mathematically expressed as follows.

Wit=Wit,  if &βi≥0Wit  Wreg,itκelev,  &if βi<0	(9)

where (κelev) is an elevation factor that scales the recalibration operation. This recalibration process tends to accentuate underlying noise to a very noticeable amount. To reduce noise that may have been introduced, we subject the recalibrated signal (Wit) to smoothing via the Savitzky-Golay filter. This filtering process is performed conditionally based on the slope (βi) as well as the post-smoothing factor (λpost) in the following manner.

Wit=SGFilterWit, wpost, p,  if &βi<βthresh and λpost>0Wit,  &otherwise		(10)

Here, the threshold (βthresh) governs the extent to which the post-calibration smoothing is applied only when the condition (βi<βthresh) holds true, and the new segment (wpost) is expressed as follows.

wpost=Tiλpost	(11)

where (λpost) is a tunable parameter controlling the degree of post-recalibration smoothing. After all windows have been processed and smoothed, the global signal can be reconstructed to finally obtain the health indicator (HI). However, since each window with a negative slope (βi) has been processed locally with no context to neighboring segments, there may be discontinuities at the boundaries between adjacent segments. Continuous transitions can be made by applying vertical adjustments to each segment of the reconstructed signal to obtain (sit) which can be mathematically defined as follows.

sit=Wit+si -1ti-1+Ti-1-1-Wit0,  &i>0Wit,  &i=0	(12)

Parameter selection

The SASD algorithm's performance is highly sensitive to the choice of its tunable parameters. After extensive manual analysis, the following parameters were chosen for optimal performance: (λwin=100), (λpre=45), (βthresh=0.4), (κelev=20.1), (λpost=3).

One important observation crucial to extracting features correctly is picking the right value for (λwin). Smaller values of (λwin) lead to larger window sizes, which can obscure underlying trends. There is a possibility of ending up with a flat regression slope due to a segment having both increasing and decreasing trends. Conversely, larger values of (λwin) result in smaller window sizes, which lead to overemphasized local trends, amplified noise in the resulting signal and is a sub-optimal approach overall. Thus, a dynamically scaled window size is needed with a tried and tested minimum bound of 10, as show in Eqn. 5. A visual representation of the same can be seen as illustrated in Fig. 2. Following signal reconstruction, health indicators derived from this algorithm roughly estimate a linear degradation process across all bearings. Taking this into consideration, we can construct an estimate of the remaining useful life (RUL) of a bearing by subtracting the normalized values of (HI) between 0 and 1 in the following manner.

RUL=1-HI-minHImaxHI-minHI	(13)

This equation is derived based on the assumption of a linear degradation process, where the health indicator degrades monotonically over time. Here, the health indicator is normalized between 0 (healthy state) and 1 (maximum degradation), represented mathematically as HI-minHImaxHI-minHI. We then subtract this normalized value from 1 to indicate the decreasing trend of the Remaining Useful Life.

Figure 2: Illustration depicting the impact of λwin on signal trends. A smaller λwin obscures trends (TOP), while the lower plot with a larger λwin amplifies noise (BOTTOM).

Extracted health indicators for bearings subjected to operating condition 1 are illustrated in Fig. 3. The extracted health indicators exhibit a strong upward trend indicating degradation of the ball bearing setups and ultimately the degradation of the machine.

Figure 3: Extracted health indicators of bearings operating under condition 1.

Dataset preparation and model training

The Slope Adaptive Signal Decomposition (SASD) algorithm is employed to extract the health indicator (sit) from the pre-computed time domain statistical features. Upon detailed manual analysis, it was observed that the signals derived using the Impulse Factor extracted from the horizontal accelerometer yielded the most informative and high-quality features making it suitable for further predictive analysis tasks relevant to the remaining useful life (RUL) of ball bearings. A sample of the same is illustrated in Fig. 4.

Figure 4: Illustration of the SASD signal sample extracted from the Impulse Factor of Bearing 1 under Condition 3, for both horizontal (left) and vertical (right) accelerometers.

For the purpose of training algorithms, we employ a range of modelling techniques.

- Gaussian Process Regression (GPR): A non-parametric Bayesian approach that is useful for modeling complex, nonlinear relationships and providing uncertainty estimates for RUL predictions.

- Long Short-Term Memory (LSTM): For capturing long-range dependencies in sequential data.

- Stacked LSTM networks: To enhance the capacity of standard LSTMs by stacking multiple layers, allowing for more complex feature extraction and improved learning of intricate patterns in the data.

- Gated Recurrent Units (GRU): Similar to LSTMs but with a simplified architecture reducing the number of parameters, leading to faster training times.

- LSTM-GRU hybrid networks: To combine the strengths of both LSTM and GRU architectures.

To model data using Gaussian Process Regression (GPR), we first enhance the quality of the input data by fitting a Logistic Growth model using ordinary least squares (OLS) to the health indicator for smoothing and representing major trends, eliminating any noise present. The logistic Growth function can be mathematically defined as follows.

yt=L1+e-kt-t0	(14)

where (yt) is the predicted value at time (t), (L) is the carrying capacity of the curve, (k) is the growth rate and (t0) is the time at which y is halfway to its inflection point.

The RUL signals used for the deep learning algorithms [17][18][19] are directly constructed from the health indicator obtained from the proposed SASD algorithm with no additional preprocessing. All extracted bearing signals are partitioned into 3 groups of train-test sets with split ratios 1:3, 1:1 and 3:1, and are evaluated accordingly.

Method validation

The performance of trained models are assessed using standard error metrics, such as root mean squared error (RMSE) as defined below.

RMSE=1ni=1nyi-yi2	(15)

where (n) represents the number of samples and (yi-yi) is the deviation of the predicted RUL datapoint from the actual RUL datapoint at time instant (i), mean absolute error (MAE) as defined below.

MAE=1ni=1nyi-yi	(16)

normalized root mean squared error (NRMSE) defined as follows.

NRMSE=1ni=1nyi-yi21ni=1nyi	(17)

Some metrics from are also used to comprehensively assess the performance of the proposed SASD algorithm. The relative percentage error (Er%) derived from the original work[1].

Eri=ActRULi-RULiActRULi×100%	(18)

in which (ActRULi) and (RULi) represent the real RUL and the estimate RUL if the (ith) bearing. The exponential transformed accuracy (see Fig. 5) is then adopted to reveal the quality of predictions based on its underestimate or overestimate in the following manner.

ETAi=e-ln0.5∙Ei5,  &Eri≤0e+ln0.5∙Ei20,  &Eri>0		(19)

after which the final score of the RUL estimation is computed using their proposed formula.

Score=1βi=1βETAi	(20)

where (β) is the number of bearings for a given operating condition. The final MAE for a model is computed as the average of all bearings for a given model, similarly NRMSE for a model is computed as the average of NRMSE for all bearings for that model.

Figure 5: Exponential Scoring function

The aforementioned metrics are computed for all architectures and for each bearing across the three split ratios. The  results are then averaged to obtain the final score. Table 3 lists the evaluation scores for 6 different models across bearings from three operating conditions. It is clear that the GRU model outperforms other architectures in majority of the bearings. This can be seen for all bearings operating under condition 1, condition 2 and in condition 3. Under condition 1, LSTM outperforms other architectures for bearing 1-2 and 1-6 in terms of relative percentage error (Er%) as well as the average of exponential transformed accuracy across all bearings under that condition. LSTM + GRU outperforms GRU for only bearing 2-4 in terms of relative percentage error (Er%) equal to (0.57%) and has considerably lower scores for the remaining metrics.

It is noteworthy that the GRU performs marginally better than the LSTM in general, relative to other architectures. This outcome can be justified by multiple factors linked to the architecture of the GRU as well as the type of data it excells at modelling compared to the LSTM and the hybrid LSTM-GRU architecture. Unlike raw vibration signals, the features extracted by the SASD algorithm are relatively shorter, which allows GRU to model it more effectively than LSTM and hybrids that include LSTM which needs longer dependencies to fully utilize its memory cell structure. The hybrid LSTM-GRU as well as the Stacked LSTM architectures introduce a larger number of trainable paramerters, leading to increased risk of overfitting. This additional increase in structural complexity of the model did not significantly improve trend modelling, making GRU a more efficient choice.

The RMSE scores can also be visualized for each bearing to compare the relative potency between architectures as illustrated in Fig. 6. Bearing 2-7 has turned out to be the only exception in this study due to poor quality of extracted health indicator (HI) resulting in much higher relative percentage errors for all models. Due to this reason the computation of Score, MAE and NRMSE for models estimating RUL of bearings operating in condition 2 will not make use of relative percentage errors computed for this bearing. It can also be observed that the GRU, LSTM, LSTM-GRU hybrid and Stacked LSTM architectures perform worse than expected compared to simpler modelling methods such as GPR and Polynomial Regression for bearing 2-7. This is due to the inability of deep learning architectures to learn intricate patterns from this specific signal as opposed to the simpler GPR and Polynomial regression capturing general trends and performing better than expected.

Figure 6: RMSE of HI prediction for all Pronostia bearings

TABLE 3: Performance Metrics of models using the proposed SASD algorithm in different conditions

|  | Method | Er% |  |  |  |  |  |  | Score | MAE | NRMSE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bearing 1-1 | Bearing 1-2 | Bearing 1-3 | Bearing 1-4 | Bearing 1-5 | Bearing 1-6 | Bearing 1-7 |  |  |  |
| Condition 1 | GRU | 0.01% | 0.65% | 0.08% | -0.37% | 0.08% | 0.52% | -0.01% | 0.9952 | 0.0037 | 0.0086 |
|  | LSTM | 0.48% | 0.13% | -0.20% | 0.36% | -0.09% | -0.07% | 0.04% | 0.9967 | 0.0060 | 0.0154 |
|  | LSTM + GRU | -1.00% | 2.47% | 0.56% | 0.80% | 0.12% | 0.51% | -0.17% | 0.9838 | 0.0078 | 0.0168 |
|  | Stacked LSTM | -1.04% | 0.46% | -0.94% | 0.26% | 0.83% | 0.10% | -0.17% | 0.9905 | 0.0101 | 0.0307 |
|  | GPR | -1.76% | -0.39% | -1.05% | -1.03% | -1.44% | 1.26% | -1.79% | 0.8843 | 0.0476 | 0.1054 |
|  | Poly Reg | -7.95% | -17.62% | -10.63% | 2.61% | -10.53% | 3.57% | -13.41% | 0.3435 | 0.0993 | 0.2134 |
|  |  | Bearing 2-1 | Bearing 2-2 | Bearing 2-3 | Bearing 2-4 | Bearing 2-5 | Bearing 2-6 | Bearing 2-7 | Score | MAE | NRMSE |
| Condition 2 | GRU | 0.35% | 0.90% | 0.16% | 0.78% | 0.07% | 0.24% | -7.16% | 0.9855 | 0.0083 | 0.0206 |
|  | LSTM | 1.98% | 2.38% | 1.65% | 0.87% | 0.25% | 0.50% | -9.02% | 0.9567 | 0.0161 | 0.0389 |
|  | LSTM + GRU | 2.22% | -1.37% | 3.83% | 0.57% | 1.44% | 3.23% | -7.78% | 0.9442 | 0.0197 | 0.0395 |
|  | Stacked LSTM | 1.08% | 2.81% | 3.25% | 1.76% | 2.50% | -1.86% | -6.54% | 0.9463 | 0.0252 | 0.0666 |
|  | GPR | 0.98% | -1.74% | 0.46% | -12.10% | 3.79% | -1.51% | -10.06% | 0.7914 | 0.0573 | 0.1381 |
|  | Poly Reg | 3.28% | -5.58% | -18.31% | -39.66% | -12.99% | -10.46% | -9.97% | 0.1445 | 0.1503 | 0.3583 |
|  |  | Bearing 3-1 | Bearing 3-2 | Bearing 3-3 |  |  |  |  | Score | MAE | NRMSE |
| Condition 3 | GRU | 1.30% | 0.10% | 0.90% |  |  |  |  | 0.9738 | 0.0107 | 0.0335 |
|  | LSTM | 0.26% | -0.15% | 2.28% |  |  |  |  | 0.9727 | 0.0173 | 0.0625 |
|  | LSTM + GRU | -0.52% | 0.68% | -1.69% |  |  |  |  | 0.9316 | 0.0138 | 0.0317 |
|  | Stacked LSTM | -1.44% | 1.15% | 4.25% |  |  |  |  | 0.9553 | 0.0267 | 0.0750 |
|  | GPR | -0.93% | -0.43% | 7.53% |  |  |  |  | 0.9312 | 0.0650 | 0.1449 |
|  | Poly Reg | 1.03% | 0.80% | -19.21% |  |  |  |  | 0.4482 | 0.1122 | 0.2571 |

Conclusion and Future Perspectives

This study has introduced the Slope Adaptive Signal Decomposition algorithm for extracting smoothed monotonic health indicators for signals in the PRONOSTIA dataset. The extracted features effectively capture degradation trends by balancing noise reduction and trend preservation and show promising result. While the method performed well on most bearings, it performed sub-optimally on bearing 2_7 where it failed to extract meaningful health indicators due to highly irregular degradation patterns. The GRU based model has broadly outperformed other architectures due to its ability to effectively capture long-term dependencies in relatively shorter sequences as opposed to the LSTM, Stacked LSTM and LSTM-GRU hybrid architectures. Future Directions include extending SASD to other platforms similar to PRONOSTIA for benchmarking its effectiveness at generalizing to more complex degradation patterns, as well as addressing its inability to account for degradation patterns as seen within this study. Further work can be done on analyzing the effectiveness of this method on different modalities such as acoustic, thermal and multi-sensor fusion approaches for a more conprehensive degradation monitoring system.

Limitations

The SASD Algorithm has certain limitations that affect the generalizability of trend extraction from signals. Careful tuning of input parameters (window factor, elevation factor, smoothing factors and threshold for slope detection is needed in order to balance noise reduction while maintaining trend preservation. Sub-optimal parameters may lead to over-smoothing leading to loss of important information or amplification of noise, inversely affecting the reliability of the health indicator. SASD also assumes a gradual monotonic degradation trend. In cases where failure modes involve sudden anamolies, this algorithm may not perform satisfactorily as discussed for bearing 7 under operating condition 2. The performance of SASD has only been benchmarked against the PRONOSTIA bearing dataset and its effectiveness for other mechanical components or modalities remains unverified.

Ethics statements

Not Applicable

CRediT author statement

Dev Bhanushali: Conceptualization, Methodology, Formal analysis, Data curation, Writing – original draft, Pooja Kamat: Conceptualization, Methodology, Writing – review & editing, Resources, Supervision, Funding acquisition

Harsh Dhiman: Conceptualization, Methodology,Review, Supervision

Acknowledgments

Funding: This work was supported by the Research Support Fund (RSF) of Symbiosis International (Deemed University), Pune, India.

Declaration of interests

☒ The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

Supplementary material and/or additional information [OPTIONAL]

None

References

[1]	P. Nectoux et al., “PRONOSTIA : An experimental platform for bearings accelerated degradation tests,” IEEE International Conference on Prognostics and Health Management, PHM’12., Jun 2012, Denver, Col- orado, United States., pp. 1–8, 2012.

[2]	M. Kostrzewski and R. Melnik, “Condition Monitoring of Rail Transport Systems: A Bibliometric Performance Analysis and Systematic Literature Review,” Sensors 2021, Vol. 21, Page 4710, vol. 21, no. 14, p. 4710, Jul. 2021, doi: 10.3390/S21144710.

[3]	M. M. U. Z. Siddiqui and A. Tabassum, “Condition-based monitoring techniques and algorithms in 3d printing and additive manufacturing: a state-of-the-art review,” Progress in Additive Manufacturing 2024, pp. 1–48, Nov. 2024, doi: 10.1007/S40964-024-00816-5.

[4]	A. Kumar, C. P. Gandhi, H. Tang, W. Sun, and J. Xiang, “Latest innovations in the field of condition-based maintenance of rotatory machinery: a review,” Meas Sci Technol, vol. 35, no. 2, p. 022003, Nov. 2023, doi: 10.1088/1361-6501/AD0F67.

[5]	Y. Liu, L. Guo, H. Gao, Z. You, Y. Ye, and B. Zhang, “Machine vision based condition monitoring and fault diagnosis of machine tools using information from machined surface texture: A review,” Mech Syst Signal Process, vol. 164, p. 108068, Feb. 2022, doi: 10.1016/J.YMSSP.2021.108068.

[6]	S. Khaleghi, M. S. Hosen, J. Van Mierlo, and M. Berecibar, “Towards machine-learning driven prognostics and health management of Li-ion batteries. A comprehensive review,” Renewable and Sustainable Energy Reviews, vol. 192, p. 114224, Mar. 2024, doi: 10.1016/J.RSER.2023.114224.

[7]	W. He et al., “Progress in prediction of remaining useful life of hydrogen fuel cells based on deep learning,” Renewable and Sustainable Energy Reviews, vol. 192, p. 114193, Mar. 2024, doi: 10.1016/J.RSER.2023.114193.

[8]	S. Wang, Y. Fan, S. Jin, P. Takyi-Aninakwa, and C. Fernandez, “Improved anti-noise adaptive long short-term memory neural network modeling for the robust remaining useful life prediction of lithium-ion batteries,” Reliab Eng Syst Saf, vol. 230, p. 108920, Feb. 2023, doi: 10.1016/J.RESS.2022.108920.

[9]	S. Plakias and Y. S. Boutalis, “Fault detection and identification of rolling element bearings with Attentive Dense CNN,” Neurocomputing, 2020, doi: 10.1016/j.neucom.2020.04.143.

[10]	P. Kamat, S. Kumar, S. Patil, and K. Kotecha, “Anomaly-informed remaining useful life estimation (AIRULE) of bearing machinery using deep learning framework,” MethodsX, vol. 12, p. 102555, Jun. 2024, doi: 10.1016/J.MEX.2024.102555.

[11]	M. Paramesha, N. L. Rane, and J. Rane, “Big Data Analytics, Artificial Intelligence, Machine Learning, Internet of Things, and Blockchain for Enhanced Business Intelligence,” Partners Universal Multidisciplinary Research Journal, vol. 1, no. 2, pp. 110–133, Jul. 2024, doi: 10.5281/ZENODO.12827323.

[12]	M. Soori, B. Arezoo, and R. Dastres, “Internet of things for smart factories in industry 4.0, a review,” Internet of Things and Cyber-Physical Systems, vol. 3, pp. 192–204, Jan. 2023, doi: 10.1016/J.IOTCPS.2023.04.006.

[13]	P. Kundu, S. Chopra, and B. K. Lad, “Multiple failure behaviors identification and remaining useful life prediction of ball bearings,” J Intell Manuf, vol. 30, no. 4, pp. 1795–1807, 2019, doi: 10.1007/s10845-017-1357-8.

[14]	P. Kamat, S. Kumar, and R. Sugandhi, “Vibration-based anomaly pattern mining for remaining useful life (RUL) prediction in bearings,” Journal of the Brazilian Society of Mechanical Sciences and Engineering, vol. 46, no. 5, pp. 1–20, May 2024, doi: 10.1007/S40430-024-04872-4/METRICS.

[15]	B. Hou, D. Wang, T. Xia, Z. Peng, and K. L. Tsui, “Difference mode decomposition for adaptive signal decomposition,” Mech Syst Signal Process, vol. 191, p. 110203, May 2023, doi: 10.1016/J.YMSSP.2023.110203.

[16]	G. Zhang et al., “Optimized adaptive Savitzky-Golay filtering algorithm based on deep learning network for absorption spectroscopy,” Spectrochim Acta A Mol Biomol Spectrosc, vol. 263, p. 120187, Dec. 2021, doi: 10.1016/J.SAA.2021.120187.

[17] 	G. Rajchakit, P. Agarwal, and S. Ramalingam, Stability Analysis of Neural Networks. Springer Singapore, 2021. doi: 10.1007/978-981-16-6534-9.

[18]	N. Boonsatit, G. Rajchakit, R. Sriraman, C. P. Lim, and P. Agarwal, “Finite-/fixed-time synchronization of delayed Clifford-valued recurrent neural networks,” Advances in Difference Equations, vol. 2021, no. 1. Springer Science and Business Media LLC, Jun. 03, 2021. doi: 10.1186/s13662-021-03438-1.

[19] 	G. Rajchakit, R. Sriraman, N. Boonsatit, P. Hammachukiattikul, C. P. Lim, and P. Agarwal, “Global exponential stability of Clifford-valued neural networks with time-varying delays and impulsive effects,” Advances in Difference Equations, vol. 2021, no. 1. Springer Science and Business Media LLC, Apr. 17, 2021. doi: 10.1186/s13662-021-03367-z.

[20] 	P. Kamat, S. Kumar, and K. Kotecha, “DeepTool: A deep learning framework for tool wear onset detection and remaining useful life prediction,” MethodsX, vol. 13. Elsevier BV, p. 102965, Dec. 2024. doi: 10.1016/j.mex.2024.102965.